## 1.2.1: Power Supplies, Voltmeters, Ammeters (25 points total)

- Sketch the circuit below, indicate the voltages vac, vbc and the current i on your sketch.  Indicate on your sketch how the current and voltage measurements are being made.  (2 pts)

- What are the voltage and current measurements vbc and i with no power applied to the circuit?  (3 pts)

- What are the measured voltages vac and vbc and the measured current i with 5V applied to the circuit?  (3 pts)

- Comment on the following relative to your results.

- What is the percent difference between the applied voltage vab indicated by the voltage source and the measured voltage vab?  (2 pts)

- What is the voltage difference across the ammeter (e.g the difference between vac and the voltage vbc)?  Is the ammeter behaving like an open circuit or a short circuit?  (2 pts)

- What is the power generated by the source?  (4 pts)

- What are the measured voltages vac and vbc and the measured current i with the voltmeter leads reversed?  Comment on your results vs. expectations, especially relative to the signs of the voltages and current.  (5 pts)

- DEMO: Have a teaching assistant initial this sheet, indicating that they have observed your circuits’ operation.  (4 pts)

TA Initials: _______